# PCbug11 Command Summary

ASM addr [mne|dir]    - Call symbolic macro line assembler, with option to auto insert mnemonic or directive

BAUD [rate]               - Display/Set serial baud rate

BF addr1 [addr2] byte|word    - Block fill memory with byte or word

BL                               - Display breakpoints

BR [addr [macroname]]         - Display/Set break point [with optional Command Macro execution]

CALL addr                 - Execute the subroutine at addr

CLRM                        - Clear all Command Macros

CLS                             - Clear main window

CONTROL [parameter]           - Display/Change PCbug11 system parameters

DASM addr1 [addr2]            - Disassemble from addr1 [to addr2]

DB startaddr [endaddr]        - Display MCU Memory

DEBUG                         - Reserved word

DEFINE symbol value|address   - Define a symbol

DEFM macrnam|TRACE|AUTOSTART  - Define Command,Trace or Autostart Macro

DELM macrnam|TRACE|AUTOSTART  - Delete Command,Trace or Autostart Macro

DIR [mask]                         - Display Disk Directory

DOS [command]                 - Shell to DOS/Execute DOS command

EDITM macrnam                 - Edit a macro

EEPROM [startaddr [endaddr]]  - Display/Clear/Set EEPROM Address Range(s)

EEPROM DELAY option           - Set EEPROM erase/write programming time

EEPROM ERASE [option]         - Display/Change EEPROM Erase-before-Write

EPROM [startaddr [endaddr]]   - Display/Clear/Set EPROM Address Range(s)

EPROM DELAY option            - Set EPROM erase/write programming time

FIND byte|word addr1 addr2    - Find all occurrences of byte or word between addr1 & addr2

FIND mnemonic addr1 addr2     - Find all occurrences of mnemonic between addr1 & addr2

G [addr]                      - Start User Code Execution

HELP                          - Display Help Information

KLE                           - Kill last error message

LOADM [filename] [macroname]  - Load Macro Definitions from Default/User File

LOADS filename [loadaddr]     - Load S Record File into MCU Memory

LS symbol                     - Display symbols

LSTM [mname|TRACE|AUTOSTART]  - Display Macro Names/Definitions

MD startaddr [endaddr]        - Display MCU Memory

MM addr                       - Modify memory from addr

MOVE addr1 addr2 addr3        - Move MCU Memory between addr1 &

addr2 to addr3

MS addr byte|word [byte|word] - Set MCU Memory Byte(s) or Word(s)

MSG [string]                  - Display Message in Main Window

NOBR [address]                - Remove all/specified Break Points

PAUSE [ms]                    - Wait for any Key Press/Delay Time

PRINT                         - Display PCbug11 version number

PROTECT [startaddr [endaddr]] - Display/Clear/Set Write Protected

Address Range(s)

QUIT [Y]                      - Terminate PCbug11 session [without

confirming]

RD [T]                        - Display/Trace MCU Registers

RESET [addr]                  - MCU Hardware Reset with Existing/New

Reset Vector.

RESTART [option]              - Restart PCbug11 with Same/New Option.

RM                            - Modify MCU Registers in window

RS register value             - Set value of MCU register

S                             - Stop User Code Execution

SHELL ["command" ;P]          - Shell to DOS/ Execute DOS command

SAVEM [filename]              - Save Macro Definitions in Default/User

File

T [addr]                      - Trace User Code

TERM [X1 Y1 X2 Y2]            - Simple windowed terminal emulator

TYPE filename                 - Display Disk file in Main Window

UNDEF symbol                  - Undefine a symbol

VER                           - Display version number

VERF ERASE addr1 [addr2]      - Verify that memory contains $FF

VERF SET addr1 addr2 value    - Verify that memory contains value

VERF filename [memaddr]       - Verify S record disk file against memory

WAIT [ms]                     - Wait for ms

Special Key operations:

CTRL B                        - Send Break on COM channel

CTRL P                        - Toggle MCU Memory Write Protect/RTS

line

CTRL R                        - Attempt to re-synchronise talker