PCbug11 V3.20 Information

Command Line Ateration

----------------------

PCbug11's DOS command line has been significantly changed for this version as compared to earlier issues.

Full details are contained in the text, however, the key features are:

. Explicit declaration of macro names, baud rate and com port

. Simplification of boot type talker selection

. Simplification of directory structure by creation of new .XOO files

. Feature to allow parameters to be passed to the AUTOSTART macro

. New single file for help

Placing of .MAP files

---------------------

The PCbug11 documentation indicates that any .MAP files should  be placed in the user's working directory. In V3.20 PCbug11 will  not find the user's .MAP file unless it is in the same directory  as PCBUG11 itself.

PCbug11 V3.21 Information

S-Records of more than 8KB

--------------------------

A problem existed in earlier versions when loading or verifying  S-records of more than 8Kbytes. This occurred when an S-record  exceeded the PC internal 8Kbyte boundary. This could cause the  PC to crash or cause strange graphic effects. This has now been  remedied.

Programming 711 parts

---------------------

68HC711 parts may be programmed more quickly using PCbug11 if an  alternative crystal is used. If a crystal of 4.9152MHz is used  then the HC11 is capable of communicating at 38400 Baud. This in  turn reduces the programming time required by internal EPROM to  one quarter of the normal time.

The method requires a talker which initialises the SCI properly,  the option baud=4800 to start PCbug11, and the digit 7 as a  prefix to the bootloader type. A talker called TALK7E.BOO is  supplied to program the 711E9 part. To use this talker the  command line should be:

PCBUG11 -7E baud=4800

On detecting the presence of the 7 and the baud=4800 options  together, PCbug11 will use a default baud rate of 38400. If  either of these options are missing then PCbug11 will default to  its standard baud rate.

PCbug11 V3.22 Information

EEPROM and EPROM bugs

---------------------

A possibility existed that the EPROM and EEPROM functions would  not work with certain options. This concerns mainly options - A/D/K. Option D would allow all the normally disallowed commands  and would not program properly. K would not program EEPROM or  EPROM properly and A would not program EEPROM correctly.

This bug has been fixed on the current version.

PCbug11 V3.23 Information

This document notes the changes made to PCbug11 for revision   3.23. The changes are noted with respect to V3.22. See also  PCbug11 V3.22 Information.

CONTROL command options

-----------------------

Two options have been added to allow the location of the PPROG  (EEPROM) and EPROG (EPROM) programming registers to be moved by  the user. This may be necessary in the future as the HC11 family  develops. The address of these registers may be the same - i.e.  one register performing both tasks. The current values are  displayed using the CONTROL command on its own. The values may  be changed using the format

CONTROL PPROG addr        (addr is the address to be used)

CONTROL EPROG addr        (addr is the address to be used)

Note that PCbug11 assumes that the relevant bits in each  register maintains the same position as in the 711E9.

User Map files

--------------

This must now be placed in the user's working directory as  described in the manual

Command Line Compiler

---------------------

The crystal value used parameter entered to the command line  compiler is now given in KHz. This will allow the use of an extended range of values.

EPROM Command

-------------

An EPROM memory range error now reads 'Invalid EPROM address  range'.

EPROM/EEPROM Programming Error

------------------------------

The error displayed is now 'Programming t/out' or 'Programming  cmd' depending on the failure.

Loading and Verifying S-records

-------------------------------

An upgrade here allows users to load and verify S-record files  which contain backward discontinuous S-records. The code was  already able to handle forward jumps in the S-record but not  backward jumps. This is now allowed.

Automatic Recognition

---------------------

PCbug11 now has the capability of recognising which HC11 is in  use from the contents of the boot ROM. The boot ROM contains  information in two bytes which help to describe the part in use.

PCbug11 reads these and displays the version in use by the MCU :  label. Note that some MCUs may not be recognised correctly by  this feature